# ADMS XML → network.json: work computer setup guide

Step-by-step instructions for running the network conversion pipeline on a work (Windows) computer. End result: a folder of solver-ready network.json files — one per feeder and one per distribution substation — built from ADMS CIM XML exports.

You will need: a Windows PC, permission to install Python (or Python already installed), the three pipeline scripts, and the ADMS XML exports. The scripts use only Python’s standard library — no packages, no internet access, and no admin rights are needed once Python is installed.

# Step 1 — Install Python (once)

Open a Command Prompt (Start → type “cmd” → Enter) and check whether Python is already there:

python --version

If it prints Python 3.9 or higher, skip to Step 2. Also try py --version — some installs only register the py launcher; if so, type py wherever this guide says python.

If Python is not installed:

- Preferred on a managed work PC: install it from your software centre (Company Portal / Software Center) if listed, or ask IT to install “Python 3.12 (64-bit)”.

- Otherwise download from https://www.python.org/downloads/ → run the installer → tick “Add python.exe to PATH” on the first screen → Install. Admin rights are not required if you choose “Install for me only”.

Close and reopen Command Prompt, then re-run python --version to confirm.

# Step 2 — Copy the three scripts

Get these three files from the OneDrive folder Honours project\Individual Project\network_conversion (via OneDrive web login, or however files are transferred at work):

- cim_to_network_json.py  — converts one feeder’s XML files to one network.json

- extract_lv_network.py  — cuts out one substation’s LV network

- batch_convert.py  — runs the two scripts above over a whole folder

Create a working folder and put all three scripts in it. In Command Prompt:

mkdir C:\netconv

mkdir C:\netconv\xml_in

Copy the three .py files into C:\netconv (File Explorer drag-and-drop is fine). All three must sit in the same folder.

# Step 3 — Collect the XML exports

Put every ADMS export into C:\netconv\xml_in — feeder exports and LVNetwork exports mixed together. Subfolders are fine; file names don’t matter (each file is identified by the circuit information inside it).

- Feeder exports — one per feeder, e.g. Lexcen_202604081113.xml

- LVNetwork exports — one per substation, e.g. S 5402_LVNetwork_202604081113.xml. Without these, a feeder still converts, but its substations have no street LV or customers.

When requesting exports from the ADMS team, ask for each feeder circuit plus all of its LVNetwork circuits (they appear in the ADMS with names like “S 5402_LVNetwork”). After a first run, the report file (Step 6) lists exactly which LVNetwork exports are still missing per feeder — use it as the follow-up request list.

# Step 4 — Test on one feeder first

Before the full network, check the pipeline works with whatever is already in the folder:

cd C:\netconv

python batch_convert.py xml_in test_out --jobs 2

Expected console output (numbers will vary):

Scanning 20 XML files...

Found 1 feeders, 18 LV networks (18 matched to feeders), 0 files skipped.

[1/1] GOLDCR_8HB_LEXCEN: ok, 940 loads, 18 substations extracted

Done: 1/1 feeders converted, 18 substation LV networks extracted.

If a feeder shows MISSING LV: ... it converted fine but some of its substations had no LVNetwork XML in the input folder.

# Step 5 — Run the whole network

cd C:\netconv

python batch_convert.py xml_in net_out --jobs 6

For ~194 feeders and ~5,000 LVNetwork files, expect well under an hour with --jobs 6. The run is safe to repeat — outputs are simply overwritten — so re-run it whenever new XML exports arrive. If the PC gets sluggish, lower --jobs to 2–4.

Results land in:

net_out\feeders\<FEEDER>_network.json           (one per feeder)

net_out\substations\<FEEDER>\<SUB>_lv_network.json   (one per substation)

net_out\batch_report.csv                        (summary + gap list)

# Step 6 — Check the report

Open net_out\batch_report.csv in Excel. Key columns:

| column | what to look for |
| --- | --- |
| status | “ok”. Any “CONVERT FAILED” row shows the error in the warnings column. |
| loads | Number of customers found. 0 or very low usually means the feeder’s LVNetwork XMLs were missing. |
| missing_lv_circuits | Substations with no LVNetwork XML in the input — this is the list to send back to the ADMS team. |
| warnings | Normal entries: open_switches, parallel_edges_merged. Investigate anything else unusual. |

Spot-check one substation before trusting the batch — list a feeder’s transformers and open one extracted file:

python extract_lv_network.py net_out\feeders\GOLDCR_8HB_LEXCEN_network.json --list

# Step 7 — Copy the outputs back

Copy the whole net_out folder to OneDrive (or wherever the modelling work happens). These files are the direct network inputs for the DOE solver — each <SUB>_lv_network.json loads as netw_ejson.

Where the load data fits: the conversion does not need consumption data — customers are already placed, named nmi_<NMI>. When the NMI consumption profiles arrive, they become the solver’s forecast input (df_forecasts), keyed by those same NMI numbers. Keep them as CSV with an NMI column and they will join directly.

# Troubleshooting

| symptom | fix |
| --- | --- |
| ‘python’ is not recognized | Use py instead of python. If that also fails, reinstall Python with “Add python.exe to PATH” ticked, then reopen Command Prompt. |
| No .xml files found | Check the input folder path, and that files end in .xml (not .zip — unzip any archives first). |
| skipped: … no cim:Circuit found | That file isn’t a DMS circuit export (or is corrupted). Harmless — it is ignored. |
| unmatched LV: … feeder not in input | An LVNetwork XML was supplied but its parent feeder’s XML wasn’t. Add the feeder export and re-run. |
| CONVERT FAILED … no feeder-head node | Rare export variant without a stitchingInfo=FH marker. Convert that feeder manually with cim_to_network_json.py using --source-node (see README). |
| Very slow / PC unresponsive | Lower --jobs to 2. Antivirus scanning thousands of new files can also slow things; run into a folder excluded from sync tools if possible. |
| Paths with spaces | Wrap any path containing spaces in double quotes, e.g. python batch_convert.py “C:\my xmls” net_out |